LIBRARY IEEE;

USE IEEE.std_logic_1164.ALL;

ENTITY buf IS

PORT ( a : IN std_logic   ;

b : OUT std_logic );

END buf;

ARCHITECTURE buf OF buf IS

BEGIN

b <= a AFTER 10 ns;

END buf;

--____________________________________________________

LIBRARY IEEE;

USE IEEE.std_logic_1164.ALL;

ENTITY delay_line IS

PORT ( a : IN std_logic   ;

b : OUT std_logic );

END delay_line;

ARCHITECTURE delay_line OF delay_line IS

BEGIN

b <= TRANSPORT a AFTER 10 ns;

END delay_line;

--____________________________________________________

LIBRARY IEEE;

USE IEEE.std_logic_1164.ALL;

ENTITY delay_line_tb IS

---------------------------

END delay_line_tb;

ARCHITECTURE delay_line_tb OF delay_line_tb IS

COMPONENT delay_line

PORT (a : IN std_logic;

b : OUT std_logic);

END COMPONENT;

SIGNAL  sa     : std_logic := '0';

SIGNAL  sb     : std_logic       ;

BEGIN

dut : delay_line

PORT MAP(

a  => sa,

b  => sb);

sa <= '1' AFTER  5 ns,

'0' AFTER  8 ns,

'1' AFTER 10 ns,

'0' AFTER 25 ns,

'1' AFTER 28 ns,

'0' AFTER 30 ns,

'1' AFTER 45 ns,

'0' AFTER 48 ns;

END delay_line_tb;

--____________________________________________________

LIBRARY IEEE;

USE IEEE.std_logic_1164.ALL;

ENTITY buf_tb IS

---------------------------

END buf_tb;

ARCHITECTURE buf_tb OF buf_tb IS

COMPONENT buf

PORT (a : IN std_logic;

b : OUT std_logic);

END COMPONENT;

SIGNAL  sa     : std_logic := '0';

SIGNAL  sb     : std_logic       ;

BEGIN

dut : buf

PORT MAP(

a  => sa,

b  => sb);

sa <= '1' AFTER  5 ns,

'0' AFTER  8 ns,

'1' AFTER 10 ns,

'0' AFTER 25 ns,

'1' AFTER 28 ns,

'0' AFTER 30 ns,

'1' AFTER 45 ns,

'0' AFTER 48 ns;

END buf_tb;

--____________________________________________________