| Attribute | Example for Attribute Usage |
| --- | --- |
| # T ' base | natural ' base -> integer |
| # T ' left | integer ' left -> -2147483647 bit ' left -> '0' |
| # T ' right | integer ' right -> 2147483647 bit ' right -> '1' |
| # T ' low | # TYPE bit4 is 15 downto 0 bit4 ' low -> 0 |
| # T ' high | # TYPE bit4 is 15 downto 0 bit4 ' high -> 15 |
| # T ' pos(X) | TYPE state IS (A, B, C, D); state ' pos (B) -> 1 |
| # T ' val(N) | TYPE state IS (A, B, C, D); state ' val (2) -> C |
| # T ' succ(X) | TYPE state IS (A, B, C, D); state ' succ (A) -> B |
| # T ' pred(X) | TYPE state IS (A, B, C, D); state ' pred (C) -> B |
| # T ' leftof(X) | TYPE state IS (A, B, C, D); state ' leftof (C) -> B |
| # T ' rightof(X) | TYPE state IS (A, B, C, D); state ' rightof (C) -> D |

## Attributes for Data Types

| Attribute | Example for Attribute Usage |
| --- | --- |
| # A ' left (N) | # TYPE mem IS ARRAY (0 TO 15, 7 DOWN 0) mem ' left (1) -> 0 mem ' left (2) -> 7 |
| # A ' right (N) | mem ' right (1) -> 15 mem ' right (2) -> 0 |
| # A ' high (N) | mem ' high (1) -> 15 mem ' high (2) -> 7 |
| # A ' low(N) | mem ' low (1) -> 0 mem ' low (2) -> 0 |
| # A ' range(N) | mem ' range (1) -> 0 to 15 mem ' range (2) -> 7 downto 0 |
| # A ' reverse_range(N) | mem ' reverse_range (1) -> 15 downto 0 mem ' reverse_range (2) -> 0 to 7 |
| # A ' length(N) | mem ' length (1) -> 16 mem ' length (2) -> 8 |

### Attributes for Arrays

| Attribute | Example for Attribute Usage |
| --- | --- |
| # S ' delayed (t) | # carry ' delayed (20 ns) |
| # S ' stable (t) | # clock ' stable (3 ns) |
| # S ' transaction | # data ' transaction |
| # S ' event | # data ' event |
| # S ' active | # reset ' active |
| # S ' quiet | # carry ' quiet |
| # S ' last_event | # reset ' last_event |
| # S ' last_active | comput ' last_active |
| # S ' last_value | comput ' last_value |

### Attributes for Signals